|  | Cairo University Faculty of Engineering Aerospace Department |  |
| --- | --- | --- |

AER 645: Finite Element method in Fluid Mechanics

Assignment #1 on: Pipe-Flow Network

Submitted To: Professor: Amr Guaily

| Name: | Ibrahim Thabet Allam | Code: 202510029 Date: 22/2/2026 |
| --- | --- | --- |

Global Matrix Symbolically

Connectivity matrix is made in excel file called : Connectivity_matrix.xlsx

Main code reads from the excel file and solve the problem

import numpy as np

import pandas as pd

from scipy.sparse.linalg import gmres, lsqr, bicg, tfqmr

import matplotlib.pyplot as plt

import matplotlib as mpl

#Plot Style

# ======================================================

# PLOT STYLE

# ======================================================

mpl.rcParams.update({

"font.family": "serif",

"font.serif": ["Times New Roman"],

"axes.labelsize": 14,

"axes.titlesize": 16,

"legend.fontsize": 11,

"xtick.labelsize": 12,

"ytick.labelsize": 12,

"figure.dpi": 120,

"axes.grid": True,

"grid.linestyle": "--",

"grid.alpha": 0.6,

"lines.linewidth": 2.2,

})

#------- Input data-------#

mu = 1e-5

d = 0.1

E = 16

NN = 14

e= 2

#-------------Reading Connectivity--------------#

data = pd.read_excel("Connectivity_matrix.xlsx")

connectivity = data[['Node1','Node2']].to_numpy().astype(int)

#-------------Defining Pipe lengths-------------#

L = np.ones(E)*10

L[1]=20

L[5]=20

L[3]=14.4

#-------------Defining Pipe conductance-------------#

K_elem = np.pi*d**4/(128*mu*L)

#----------Defining Global and local matrix---------#

KG = np.zeros((NN,NN))

for i in range(E):

node1 = connectivity[i,0]-1

node2 = connectivity[i,1]-1

k = K_elem[i]

#local matrix assembly

KG[node1,node1] +=k

KG[node1,node2] -= k

KG[node2,node1] -= k

KG[node2,node2] += k

KG_old = KG.copy()

# --------- RHS VECTOR ----------

RHS = np.zeros(NN)

RHS[0] = 5

RHS[3] = -0.71428

RHS[4] = -0.71428

RHS[5] = -0.71428

#Boundary Conditions

P = np.zeros(NN)

BNode = [11,12,13,14]

for n in BNode:

P[n-1] = 1

# --- MODIFYING RHS  ---

for bn in BNode:

bn -= 1

for i in range(NN):

RHS[i] -= KG[i,bn]*P[bn]

# --- Modify K matrix ---

KG_BNode = []

for bn in BNode:

KG_BNode.append(KG[bn-1,bn-1])

KG[:,[bn-1 for bn in BNode]] = 0

KG[[bn-1 for bn in BNode],:] = 0

for i,bn in enumerate(BNode):

KG[bn-1,bn-1] = KG_BNode[i]

RHS[bn-1] = KG_BNode[i]*P[bn-1]

# --------- DIRECT SOLVER ----------

P_exact = np.linalg.solve(KG,RHS)

q_exact = KG_old @ P_exact

print("\nExact Pressures [bar]:")

print(P_exact)

print("\nExact Node Flows [m3/s]:")

print(q_exact)

#GMRES

P_gmres,_ = gmres(KG,RHS)

#LSQR

P_lsqr = lsqr(KG,RHS)[0]

#BiCG

P_bicg,_ = bicg(KG,RHS)

#TFQMR

P_tfqmr,_ = tfqmr(KG,RHS)

#Relative Errors

Error1 = (P_gmres-P_exact)/P_exact*100

Error2 = (P_lsqr-P_exact)/P_exact*100

Error3 = (P_bicg-P_exact)/P_exact*100

Error4 = (P_tfqmr-P_exact)/P_exact*100

# ======================================================

# ERROR PLOTS

# ======================================================

plt.figure(figsize=(11,8))

titles = [

"GMRES Method Error",

"LSQR Method Error",

"BiCG Method Error",

"TFQMR Method Error"

]

errors = [Error1, Error2, Error3, Error4]

for i in range(4):

ax = plt.subplot(2,2,i+1)

ax.plot(range(1, NN+1), errors[i], marker='o',color ='r')

ax.set_title(titles[i])

ax.set_xlabel("Node Number")

ax.set_ylabel("Percentage Error (%)")

# symmetric y-axis around zero for clarity

max_err = np.max(np.abs(errors[i]))

ax.set_ylim(-1.1*max_err, 1.1*max_err)

plt.tight_layout()

plt.show()

# Result of the Code:

Exact Pressures [bar]:

[221.19867217  99.274332    40.3728745   10.92214574  35.19583333

51.29967801  10.57380233  19.79926125  32.42740936  57.61069818

1.           1.           1.           1.        ]

Exact Node Flows [m3/s]:

[ 5.00000000e+00  2.77555756e-16  0.00000000e+00 -7.14280000e-01

-7.14280000e-01 -7.14280000e-01  0.00000000e+00  2.22044605e-16

4.44089210e-16  4.44089210e-16 -2.34976462e-01 -4.61403289e-01

-7.71344675e-01 -1.38943557e+00]